STEP1: Connect to the device via console

Use a terminal emulation software such as PuTTY and connect to the console of the switch. You will get the initial command prompt “Switch>”

Type “enable” and hit enter. You will get into privileged EXEC mode (“Switch#”)

Now, get into Global Configuration Mode:

Switch# configure terminal
Switch(config)#

Note: The switch will not ask you for a password when entering into Privileged EXEC mode (i.e after typing “enable”) if it has the default factory configuration. See Step 3 below about setting up a password for the Privileged EXEC mode.

STEP2: Set up a hostname for the particular switch to distinguish it in the network

Switch(config)# hostname BSCS-6
BSCS-6 A Morning (config)#

STEP3: Configure an administration password (enable secret password)

Purpose: Protects access to privileged EXEC mode (higher-level commands).

Use Case: Ensures only authorized users can make configuration changes.

Two Variants:

enable password: Basic password.

enable secret: Encrypted password, preferred for better security.

access-switch1(config)# enable password 123

access-switch1(config)# enable secret 123

STEP4: Configure a password for Telnet and Console access

Console Password

Purpose: Controls access to the switch via the physical console port.

Use Case: For administrators physically accessing the switch.

access-switch1(config)# line console 0
access-switch1(config-line)# password 123

access-switch1(config-line)# login
access-switch1(config-line)# exit

VTY (Virtual Terminal Lines) Password

Purpose: Controls access to the switch via SSH or Telnet (remote access).

Use Case: For administrators accessing the switch remotely.

access-switch1(config)# line vty 0 2

access-switch1(config-line)# password 123
access-switch1(config-line)# login
access-switch1(config-line)# exit

STEP5: Define which IP addresses are allowed to access the switch via Telnet

access-switch1(config)# ip access-list standard abc
access-switch1(config-std-nacl)# permit 10.0.0.7

access-switch1(config-std-nacl)# permit 10.1.1.5

access-switch1(config-std-nacl)# deny 10.0.0.6 
access-switch1(config-std-nacl)# exit

!Apply the access list to Telnet VTY Lines
access-switch1(config)# line vty 0 3
access-switch1(config-line)# access-class abc in
access-switch1(config-line)# exit
access-switch1(config)#

STEP6: Assign IP address to the switch for management

!Management IP is assigned to Vlan 1 by default
access-switch1(config)# interface vlan 1
access-switch1(config-if)# ip address 10.0.0.5 255.0.0.0
access-switch1(config-if)# no shutdown
access-switch1(config)#exit

STEP7: Assign default gateway to the switch

access-switch1(config)# ip default-gateway 10.1.1.254

STEP8: Disable unneeded ports on the switch

! This step is optional but enhances security
! Assume that we have a 48-port switch and we don’t need ports 25 to 48

access-switch1(config)# interface range fa 0/25-48
access-switch1(config-if-range)# shutdown
access-switch1(config-if-range)# exit
access-switch1(config)#

STEP9: Configure Layer2 VLANs and assign ports to the them

By default, all physical ports of the switch belong to the native VLAN1. One of the most important functions of an Ethernet switch is to segment the network into multiple Layer2 VLANs (with each VLAN belonging to a different Layer3 subnet).

MORE READING:  How to configure a Cisco Layer 3 Switch-InterVLAN Routing Without Router

In order to do the above Layer2 segmentation you need to create additional VLANs from the default VLAN1 and then assign physical ports to these new vlans. Let’s create two new vlans (VLAN2 and VLAN3) and assign two ports to each one.

! First create the Layer2 VLANs on the switch

access-switch1(config)# vlan 2
access-switch1(config-vlan)# name TEACHERS
access-switch1(config-vlan)# exit

access-switch1(config)# vlan 3
access-switch1(config-vlan)# name STUDENTS
access-switch1(config-vlan)# exit

! Now assign the physical ports to each VLAN. Ports 1-2 are assigned to VLAN2 and ports 3-4 to VLAN3

access-switch1(config)# interface range fa 0/1-2
access-switch1(config-if-range)# switchport mode access
access-switch1(config-if-range)# switchport access vlan 2
access-switch1(config-if-range)# exit

access-switch1(config)# interface range fa 0/3-4
access-switch1(config-if-range)# switchport mode access
access-switch1(config-if-range)# switchport access vlan 3
access-switch1(config-if-range)# exit

STEP10: Save the configuration

access-switch1(config)# exit
access-switch1# wr

The above command to save the configuration can also be accomplished with  copy run start

The above are some steps that can be followed for basic set-up of a Cisco switch. Of course there are more things you can configure (such as SNMP servers, NTP, AAA, Vlan trunking protocol, 802.1q Trunk ports, Layer 3 inter-vlan routing etc) but those depend on the requirements of each particular network.

Some Useful “Show” Commands

After configuring the basic steps above, let’s see some useful commands to monitor your configuration or troubleshoot possible problems:

access-switch1# show run  (Displays the current running configuration)
access-switch1# show interfaces  (Displays the configuration of all interfaces and the status of each one)
access-switch1# show vlan  (Displays all vlan numbers, names, ports associated with each vlan etc)
access-switch1# show interface status  (Displays status of interfaces, speed, duplex etc)
access-switch1# show mac address-table  (Displays current MAC address table and which MAC address is learned on each interface)